Faculty of Engineering and Technology

Department of Computer Science

COMP432 - Computer Security

Securing Layer 2 Networks

STP-DHCP Reconvergence Attacks and Defense Strategies

A Comprehensive Study of Layer 2 Network Attacks and Defense Strategies

| Prepared By: | Osama Abu jarad |
| --- | --- |
| Student ID: | 1202883 |
| Instructor: | Dr. Hafez Barghouthi |
| Submission: | Initial report topic submission |
| Date: | April 2026 |

Recommended short topic name: Layer 2 Reconvergence Security

Abstract

Layer 2 networks are often treated as trusted internal zones, yet many attacks begin before traffic reaches firewalls or application security controls. This report studies how Spanning Tree Protocol (STP), Rapid STP (RSTP), DHCP, ARP, and HTTPS interact during normal switching and during topology reconvergence. The main focus is the short but important period after a link or switch change, when ports move between states, forwarding paths change, and binding tables may be incomplete or stale. Attackers can exploit this window through rogue DHCP servers, DHCP starvation, ARP spoofing, BPDU manipulation, and man-in-the-middle positioning. The proposed defense strategy combines RSTP, PortFast edge configuration, BPDU Guard, Root Guard, DHCP Snooping, Dynamic ARP Inspection (DAI), IP Source Guard, port security, segmentation, and monitoring. HTTPS remains necessary for confidentiality, but it must be supported by Layer 2 controls because encryption alone cannot prevent local traffic redirection.

Introduction

A switched LAN depends on several automatic trust decisions. STP prevents loops, DHCP assigns addressing information, and ARP maps IP addresses to MAC addresses. These protocols were designed for availability and convenience, not for hostile access networks. In a campus, lab, or enterprise floor, a single unauthorized device can advertise itself as a better bridge, answer DHCP requests faster than the real server, or poison ARP caches so that users send traffic through the attacker. The study is therefore named Securing Layer 2 Networks: STP-DHCP Reconvergence Attacks and Defense Strategies, because it connects the protocol behavior to practical defense controls.

1. Current STP-DHCP Integration Challenges

Separate control planes: STP/RSTP changes forwarding topology, while DHCP Snooping and IP Source Guard depend on accurate per-port and per-VLAN bindings.

Reconvergence timing: classic STP may delay forwarding for tens of seconds, while RSTP improves recovery but still requires correct edge and trunk design [3].

Binding accuracy: DHCP Snooping creates trusted state from DHCP exchanges, but static hosts, sleeping devices, virtual machines, and moved ports can leave gaps.

Trust boundary mistakes: an uplink should be trusted for DHCP/DAI, but an access port should normally remain untrusted; one wrong trust setting can permit spoofed control traffic.

Protocol mismatch: HTTPS protects application payloads, but it does not stop a local attacker from forcing gateway, DNS, DHCP, or ARP redirection before TLS is established.

2. Security Vulnerabilities During Reconvergence

| Attack | Reconvergence Weakness | Impact | Primary Defense |
| --- | --- | --- | --- |
| Rogue DHCP | Client broadcasts may reach an attacker before the legitimate server path is stable. | Wrong gateway/DNS, traffic interception, outage. | DHCP Snooping, trusted uplinks only. |
| ARP spoofing / MITM | Caches and inspection bindings can be stale or missing after a port/path change. | Session hijack, credential exposure, traffic manipulation. | DAI with DHCP bindings or ARP ACLs [5]. |
| BPDU spoofing | A hostile switch may attempt to become root or alter the tree during topology change. | Traffic redirection, loops, unstable LAN. | BPDU Guard, Root Guard, correct bridge priority [4]. |
| DHCP starvation | Repeated spoofed clients exhaust the address pool while users reconnect. | Denial of service, easier rogue DHCP success. | Port security, rate limits, DHCP Snooping. |
| MAC flooding | Forwarding database churn can increase during link changes. | Unknown unicast flooding and sniffing opportunity. | Port security, storm control, segmentation. |

3. Proposed Security Solutions

The best defense is layered. RSTP should replace legacy STP where supported because it provides faster convergence and can interoperate with legacy 802.1D bridges when necessary [3]. Edge ports should use PortFast or equivalent edge behavior, but only for end-host ports. BPDU Guard should shut down an edge port if it receives a BPDU, because that indicates a possible unauthorized switch or a cabling mistake [4]. Root Guard and Loop Guard should protect upstream topology stability.

For address integrity, DHCP Snooping should be enabled globally and per VLAN, with only server-facing uplinks trusted. DAI should validate ARP requests and replies against the DHCP Snooping binding database, while ARP ACLs should cover static hosts [5]. IP Source Guard should permit IP traffic only when the source IP and MAC match a valid binding, helping block spoofed source traffic on Layer 2 access ports [6]. Add 802.1X/NAC, port security, VLAN segmentation, storm control, and centralized logging to make the controls operational rather than only theoretical.

4. Implementation Strategy

Map the network first: root bridge location, VLANs, trunks, DHCP server/relay ports, static hosts, voice/Wi-Fi/VM ports, and critical services.

Enable RSTP/MSTP and set deliberate bridge priorities so the core/distribution layer remains the root. Mark only end-host ports as edge/PortFast.

Turn on DHCP Snooping for selected VLANs, trust only uplinks toward DHCP servers or relays, and enable rate limits on access ports.

After bindings are stable, enable DAI per VLAN and add ARP ACLs/static bindings for servers, printers, cameras, or devices without DHCP.

Enable IP Source Guard on user-facing access ports, then monitor drops, err-disable events, DHCP binding changes, and ARP inspection logs before wider rollout.

5. Performance and Compatibility Considerations

Security controls must not be enabled blindly. DAI performs validation work and is commonly paired with ARP rate limiting to avoid CPU pressure [5]. IP Source Guard consumes hardware filtering resources and can deny all IP traffic if no binding exists for a port [6]. Older switches may support only legacy STP, and mixed environments can force RSTP to interoperate at slower behavior. DHCP Option 82, static addressing, trunk ports, wireless controllers, IP phones, and virtualization hosts require careful testing. A staged deployment with logging-only observation where possible reduces accidental outages.

6. Risk Assessment and Mitigation

| Risk | Level | Reason | Mitigation |
| --- | --- | --- | --- |
| Rogue DHCP | High | Easy to launch from an access port. | DHCP Snooping, port trust policy, alerts. |
| ARP MITM | High | ARP has no built-in authentication [1]. | DAI, IP Source Guard, HTTPS/HSTS. |
| BPDU attack | Medium-High | Can alter forwarding or create instability. | BPDU Guard, Root Guard, known root bridge. |
| Control misconfiguration | High | Security controls can block valid traffic. | Pilot VLANs, rollback plan, documented exceptions. |
| Legacy compatibility | Medium | Old devices may not support every feature. | Phased rollout and vendor-specific testing. |

7. Future Enhancements and Emerging Solutions

Future work should extend the design beyond IPv4 access switching. IPv6 networks need RA Guard, DHCPv6 Guard, Neighbor Discovery inspection, and source validation. 802.1X with dynamic VLAN assignment can prevent unknown devices from reaching production segments at all. MACsec can protect traffic on sensitive switch-to-switch links. Software-defined access and EVPN/VXLAN fabrics can reduce flat Layer 2 blast radius, while telemetry can correlate STP topology changes, DHCP binding updates, ARP inspection drops, and certificate warnings into a single incident view.

8. Conclusion

Layer 2 security is a foundation for every higher-layer defense. STP/RSTP keeps the LAN loop-free, DHCP makes addressing manageable, and ARP enables local delivery, but together they create trust assumptions that attackers can abuse. The most practical solution is not one feature but a coordinated baseline: stable RSTP design, protected edge ports, DHCP Snooping, DAI, IP Source Guard, port security, segmentation, and continuous monitoring. With these controls, reconvergence becomes a managed availability event instead of an opportunity for man-in-the-middle attacks or denial of service.

9. References

[1] D. C. Plummer, RFC 826: An Ethernet Address Resolution Protocol, IETF, 1982. https://datatracker.ietf.org/doc/html/rfc826

[2] R. Droms, RFC 2131: Dynamic Host Configuration Protocol, IETF, 1997. https://www.rfc-editor.org/rfc/rfc2131

[3] Cisco, Understand Rapid Spanning Tree Protocol (802.1w). https://www.cisco.com/c/en/us/support/docs/lan-switching/spanning-tree-protocol/24062-146.html

[4] Cisco, Understand the Spanning Tree PortFast BPDU Guard Enhancement. https://www.cisco.com/c/en/us/support/docs/lan-switching/spanning-tree-protocol/10586-65.html

[5] Cisco, Configuring Dynamic ARP Inspection. https://www.cisco.com/c/en/us/td/docs/switches/lan/catalyst9500/software/release/17-13/configuration_guide/sec/b_1713_sec_9500_cg/configuring_dynamic_arp_inspection.pdf

[6] Cisco, Configuring DHCP Features and IP Source Guard. https://www.cisco.com/c/en/us/td/docs/switches/connectedgrid/cgsswitch_master/software/configuration/guide/security/Security/sec_dhcp82.html